HOT WORK PERMIT

APPROVAL FORM

This Hot Work Permit Approval Form is required for any temporary operation involving open flames or producing heat and/or sparks.  This includes, but is not limited to:  Brazing, Cutting, Grinding, And Soldering, Thawing Pipe, Torch Applied Roofing, And Welding.

INSTRUCTIONS

- Fire safety supervisor:

- Verify precautions listed at right (or do not proceed with the work).

- Retain this approval form on work site.

| DATE: |  | JOB NO. |  |
| --- | --- | --- | --- |
| LOCATION/BUILDING & FLOOR: |  |  |  |
| I verify the above location has been examined, the precautions checked on the Required Precautions Checklist have been taken to prevent fire, and permission is authorized for this work. |  |  |  |
| SIGNED ( CAMPUS FIRE SAFETY /OPERATIONS SUPERVISOR): |  |  |  |
| Farmingdale State Physical Plant |  |  |  |
| PERMIT EXPIRES | DATE |  | TIME A.M. P.M. |

THIS PERMIT IS GOOD

FOR ONE DAY ONLY UNLESS APPROVED OTHERWISE!

REQUIRED PRECAUTIONS
CHECKLIST

Available sprinklers, hose streams and extinguishers are in service/operable.

Hot Work equipment in good repair.

Requirements within 10 m (35 ft.) of work

Flammable liquids, dust, lint and oily deposits removed.

Explosive atmosphere in area eliminated.

Floors swept clean.

Combustible floors wet down, covered with damp sand or fire-resistive sheets.

Remove other combustibles where possible.  Otherwise protect with fire-resistant tarpaulins or metal shields.

All wall and floor openings covered.

Fire resistant tarpaulins suspended beneath work.

Work on walls or ceilings/enclosed equipment

Construction is noncombustible and without combustible covering or insulation.

Combustibles on other side of walls moved away.

Danger exist by conduction of heat into another area.
  Enclosed equipment cleaned of all combustibles.

Containers purged of flammable liquids/vapors.

Pressurized vessels, piping and equipment removed from service, isolated and vented.

Fire watch/hot work area monitoring

Fire watch will be provided during and for 30 minutes after work, including any coffee or lunch breaks.

Fire watch is supplied with suitable extinguishers.

Fire watch is trained in use of this equipment and in sounding alarm.

Fire watch may be required for adjoining areas above, and below.

Monitor Hot Work area for 30 minutes after job is completed.

Other precautions taken

Confined space entry permit required.

Area protected with smoke or heat detection.

Ample ventilation to remove smoke/vapor from work area.

Lockout/tagout required.

Note:  When used in accordance with NFPA 51B, this permit is to be used for, but not limited to, the following: welding, cutting, grinding, open-flame soldering, thawing pipe, and torch-applied roofing.  Copyright NFPA